Mandatory requirements.

1. Processor (CPU): Number of cores and processor model. The load is measured by reading the “busy” and “idle” time counters in /proc/stat twice, with an interval of approximately 1 second. You then calculate the ratio based on the difference between the readings. This is necessary because the counters accumulate values since boot. The program must display the load for each core and the average load.

2. Memory: Total memory, used memory, and usage percentage.

3. PCI: List all devices with their bus/device/function (BDF), vendor ID, device ID, and class, stored in an array or list.

4. USB: Vendor ID and product ID for each device. A virtual machine may not have any USB devices, and the program must still run correctly and display a message indicating that no devices were found.

5. Names: Translate the IDs using the pci.ids and usb.ids files. For example, 0x8086 translates to “Intel Corporation”. If a name cannot be found, display the raw ID instead.

6. Analysis: Sort the PCI list in two ways: alphabetically by manufacturer name and numerically by bus/device/function (BDF). Also calculate at least one summary, such as the number of devices per manufacturer or per class.

7. Output: Everything must be displayed as well-organised text, running in the terminal without a graphical user interface (GUI), because the program must work on the alderaan server.

| Requirement | Does the starter code do it? | What is missing |
| --- | --- | --- |
| 1. CPU: cores and model | Yes | Improve the formatting |
| 1. Per-core load and average | No, it only provides raw ticks | Read the values twice, wait 1 second, calculate the differences and the percentage for each core, and then calculate the average |
| 2. Memory usage and percentage | Partially (in KiB) | Convert to GiB and calculate the percentage |
| 3 and 4. PCI and USB devices stored in lists | No, it only provides methods for accessing devices by index | Create classes such as PciDevice and UsbDevice, and build lists of objects |
| 5. Device names, with the raw ID as a fallback | Partially: it prints null | Handle unknown devices (you have already encountered this bug with 1414:008a) |
| 6. Sort in two ways | No | Implement comparators by name and by bus/device/function |
| 6. Summary (by vendor or class) | No | Count and group the devices |
| 7. Readable output | No, the demo output is raw | Design the report |

| Person | Part | Main responsibilities |
| --- | --- | --- |
| Flourish | Processor (CPU) | Load calculation (two readings) |
| Eoin | Memory, USB, disk | main, output layout, README |
| Param | PCI | List, names, classes, sorting, summaries |
| Claiton | Extension + simple verdict | Performance measurement + “What is it useful for?” |

- Flourish: Processor (CPU)

Number of cores and processor model, with an alternative for ARM64 (CPU implementer and CPU part when model name is unavailable).

  - Per-core load and average: read busy and idle counters, wait 1 second, read them again, and calculate Δbusy ÷ (Δbusy + Δidle).

  - Useful extras for the verdict: frequency (cpu MHz), physical cores versus threads (cpu cores, siblings).

  - Detect virtual machines using the hypervisor flag in /proc/cpuinfo.

  - Deliverable: a “PROCESSOR” section ready to print.

- Eoin : Memory, USB, disk, and integration

  - Memory: total, used, and usage percentage, converted from KiB to GiB.

  - USB: list devices with names obtained from usb.ids. If there are none, display “no USB devices found (common in virtual machines)”.

  - disk information, including capacity and whether it is an SSD or HDD, by reading /sys/block.

  - Owner of the overall structure: Main, the visual output format (headings, columns, units), and the README with the table “information → source file”.

  - Deliverable: “MEMORY”, “USB”, and “DISK” sections, plus the complete working program.

- Param: PCI

  - Build a list of PciDevice objects (bus, device, function, vendor ID, device ID, class).

  - Resolve names using pci.ids. If a name cannot be found, display the raw ID + the type based on the class, as in 1414:008a → “unknown (3D controller)”.

  - Two sorting methods: by manufacturer name and by bus/device/function.

  - Summary: devices grouped by manufacturer and by class.

  - Function to identify the graphics card (base class 03), used by Person 4.

  - Deliverable: “PCI DEVICES” section + summaries.

- Claiton: Extension and verdict

  - Parallelism extension: run a computationally intensive task using 1, 2, 4, and 8 threads, measure the speedup, and compare it with Amdahl’s Law.

  - Verdict in plain English: “Study ✔ / Programming ✔ / Video ~ / Gaming ✘”, using the data from Persons 1, 2, and 3 and the extension’s measurements.

  - Justify every threshold used (for example, why 8 GiB is considered “sufficient for programming”).

  - Deliverable: “MEASURED PERFORMANCE” and “THIS COMPUTER IS GOOD FOR...” sections.